# SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO4:

Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL6)

Assessment Tool Weightage: 40%

Annexure A

Coding challenge

- Develop a Python program that generates a Star Topology for a given number of hosts connected to a central switch. The program should accept the number of hosts as input and display the topology either in ASCII format or a graphical representation. For each host, print the host label, the name of the central switch, and the corresponding Cat6 UTP cable length connecting the host to the switch. Verify that all hosts are connected through the central switch and explain how the star topology facilitates reliable communication in a local area network.

- Write a Python function named validate_segment(length_m) to verify whether a given UTP cable segment satisfies the IEEE

802.3 Ethernet standard, which specifies a maximum cable length of 100 meters. The function should return True for valid cable lengths and return False along with an appropriate error message for invalid lengths exceeding the permissible limit. Demonstrate the correctness of the program by executing it with at least three different test cases and interpret the results.

- Develop a Python program to simulate the operation of the Carrier Sense Multiple Access with Collision Detection (CSMA/CD) protocol in a star topology consisting of four hosts connected through a central switch. The program should simulate frame transmission attempts, detect transmission collisions, calculate random back-off intervals, and continue transmission after the waiting period. Record every collision event, transmission attempt, and back-off interval in a log file named csma_log.txt, and analyze how CSMA/CD minimizes data transmission conflicts in Ethernet networks.

- Develop a client-server application using Python's socket library to simulate communication between two hosts connected through a central switch in a star topology. The server should receive frames from the client, forward them appropriately, and maintain a simulated MAC Address Table. After each successful frame transmission, display the updated MAC address table showing the learned MAC addresses and associated ports. Explain how MAC learning and frame forwarding are performed in Ethernet switches.

- Design and implement a Python class named StarSwitch that models the basic functionality of an Ethernet switch. The class should include methods such as add_port(host_id) to connect a host, remove_port(host_id) to disconnect a host, and broadcast(frame) to transmit a frame to all connected ports except the source port. The program should maintain a record of active ports and log every broadcast operation, indicating the source host and the destination hosts that successfully receive the frame. Demonstrate the implementation using multiple hosts and explain the significance of broadcasting in Ethernet communication.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 15% | Fully understands the problem and requirements; no clarification needed | Understands most requirements with minor clarification | Partial understanding; misses some requirements | Poor understanding of the problem |
| Algorithm Design | 20% | Efficient, optimal, and well-structured algorithm | Correct algorithm with minor inefficiencies | Basic approach but not optimal | Incorrect or no clear algorithm |
| Code Quality | 15% | Clean, well-organized, readable, and properly formatted code | Mostly clean code with minor issues | Code is somewhat unclear or inconsistent | Poorly written, unreadable code |
| Functionality | 20% | Code works perfectly for all test cases | Works for most test cases | Works for limited cases | Does not run or fails major cases |
| Error Handling | 10% | Handles all edge cases and errors effectively | Handles some edge cases | Limited error handling | No error handling |
| Efficiency | 10% | Highly optimized in time and space complexity | Reasonably efficient | Some inefficiencies | Highly inefficient |
| Documentation & Comments | 5% | Clear and meaningful comments and documentation | Adequate comments | Few or unclear comments | No comments |
| Testing & Debugging | 5% | Thoroughly tested with multiple cases; no bugs | Tested with some cases | Minimal testing | No testing done |

1. Star Topology Generator

This program accepts the number of hosts, assigns a Cat6 cable length to each host, displays an ASCII star topology, and verifies that every host is connected to the central switch.

Python code:

# Star Topology Generator

n = int(input("Enter number of hosts: "))

switch = "Central Switch"

hosts = []

for i in range(1, n + 1):

length = float(input(f"Enter Cat6 cable length for Host {i} (in meters): "))

if length > 100:

print("Cable length cannot exceed 100 meters.")

length = 100

hosts.append((f"Host{i}", length))

print("\n========== STAR TOPOLOGY ==========")

print("              [Central Switch]")

print("                    |")

for host, length in hosts:

print(f"        +-----------+-----------+")

print(f"        | {host:<8} --> {switch} |")

print(f"        | Cable Length: {length} m |")

print("\n========== CONNECTION DETAILS ==========")

for host, length in hosts:

print(f"{host} --> {switch} --> Cat6 UTP: {length} meters")

# Verification

if len(hosts) == n:

print("\nVerification: SUCCESS")

print(f"All {n} hosts are connected through the central switch.")

else:

print("\nVerification: FAILED")

Output:

In a star topology, every host has a separate connection to a central switch. If one cable fails, only that host is affected. The other hosts can continue communicating, making the topology reliable and easy to troubleshoot.

2. UTP Cable Length Validation

The maximum Ethernet copper cable segment length is 100 meters. This function checks whether the given length is valid.

Python code:

def validate_segment(length_m):

if length_m < 0:

return False, "Error: Cable length cannot be negative."

if length_m <= 100:

return True, "Valid: Cable segment is within the 100-meter limit."

return False, "Error: Cable segment exceeds the maximum limit of 100 meters."

# Test cases

test_cases = [50, 100, 120]

for length in test_cases:

result, message = validate_segment(length)

print(f"Length = {length} meters")

print(f"Result = {result}")

print(message)

print("-----------------------------")

Output:

3. CSMA/CD Simulation

Important exam point: Traditional CSMA/CD applies to shared, half-duplex Ethernet. A modern switched Ethernet network normally uses full-duplex communication, where collisions do not occur. For this coding challenge, we simulate the collision behavior requested.

The program:

- Uses 4 hosts.

- Generates transmission attempts.

- Detects collisions.

- Generates a random back-off interval.

- Retries transmission.

- Records everything in csma_log.txt.

Python code:

import random

import time

hosts = ["Host1", "Host2", "Host3", "Host4"]

log_file = "csma_log.txt"

def log(message):

print(message)

with open(log_file, "a") as file:

file.write(message + "\n")

# Clear previous log

with open(log_file, "w") as file:

file.write("========== CSMA/CD LOG ==========\n")

for frame in range(1, 6):

source = random.choice(hosts)

attempt = 1

while True:

log(f"\nFrame {frame}: {source} - Attempt {attempt}")

log("Carrier Sense: Channel checked.")

# Randomly decide whether collision occurs

collision = random.choice([True, False, False])

if collision:

log("COLLISION DETECTED!")

# Random binary backoff

backoff = random.randint(1, 10)

log(f"Back-off interval = {backoff} time units")

time.sleep(0.1)

log(f"{source} waits for {backoff} time units.")

log(f"{source} retransmits the frame.")

attempt += 1

else:

log(f"{source}: Frame {frame} transmitted successfully.")

break

log("\n========== TRANSMISSION COMPLETE ==========")

log("CSMA/CD simulation completed.")

log("Log saved in csma_log.txt")

Example output

Frame 1: Host2 - Attempt 1

Carrier Sense: Channel checked.

COLLISION DETECTED!

Back-off interval = 4 time units

Host2 waits for 4 time units.

Host2 retransmits the frame.

Frame 1: Host2 - Attempt 2

Carrier Sense: Channel checked.

Host2: Frame 1 transmitted successfully.

4. Client-Server Ethernet Switch Simulation

This program uses Python's socket library.

The server represents the central switch, while the client represents a host.

The switch learns:

MAC Address → Port

and then forwards frames according to its MAC address table.

Server Program

Save as server.py:

Python code:

import socket

HOST = "127.0.0.1"

PORT = 5000

# Simulated MAC address table

mac_table = {}

server = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

server.bind((HOST, PORT))

server.listen(5)

print("Ethernet Switch Server Started")

print("Waiting for connections...\n")

while True:

connection, address = server.accept()

print("Host connected:", address)

# Receive frame

data = connection.recv(1024).decode()

if not data:

connection.close()

continue

# Frame format:

# SOURCE_MAC,DESTINATION_MAC,DATA

parts = data.split(",")

source_mac = parts[0]

destination_mac = parts[1]

message = parts[2]

# Learn source MAC address

mac_table[source_mac] = str(address[1])

print("\nFrame Received")

print("Source MAC      :", source_mac)

print("Destination MAC :", destination_mac)

print("Data            :", message)

print("\nMAC Address Table")

print("-----------------------------")

for mac, port in mac_table.items():

print(f"{mac} --> Port {port}")

# Forward frame

if destination_mac in mac_table:

print("\nDestination found.")

print("Frame forwarded to destination.")

response = "Frame forwarded successfully."

else:

print("\nDestination unknown.")

print("Frame will be broadcast.")

response = "Destination unknown - frame broadcast."

connection.send(response.encode())

connection.close()

Client Program

Save as client.py:

import socket

HOST = "127.0.0.1"

PORT = 5000

source_mac = input("Enter Source MAC Address: ")

destination_mac = input("Enter Destination MAC Address: ")

message = input("Enter message/frame data: ")

client = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

client.connect((HOST, PORT))

frame = source_mac + "," + destination_mac + "," + message

client.send(frame.encode())

response = client.recv(1024).decode()

print("\nSwitch Response:")

print(response)

client.close()

How to run

Open two terminals.

Terminal 1:

python server.py

Then open Terminal 2:

python client.py

Example:

Enter Source MAC Address: AA:BB:CC:DD:EE:01

Enter Destination MAC Address: AA:BB:CC:DD:EE:02

Enter message/frame data: Hello Host2

The server learns the source MAC address and adds it to the MAC table.

MAC Learning

When a switch receives a frame, it examines the source MAC address and records the port from which the frame arrived.

Example:

AA:BB:CC:DD:EE:01 --> Port 5001

AA:BB:CC:DD:EE:02 --> Port 5002

Frame Forwarding

- If the destination MAC is known → frame is sent only to the appropriate port.

- If the destination MAC is unknown → frame is flooded/broadcast to the relevant ports.

- The switch continues learning MAC addresses from incoming frames.

5. StarSwitch Python Class

This program creates a class representing a basic Ethernet switch.

It supports:

- add_port()

- remove_port()

- broadcast()

- Active port tracking

- Broadcast logging

Python code:

class StarSwitch:

def __init__(self):

self.active_ports = []

self.log_file = "switch_broadcast_log.txt"

# Create/clear log file

with open(self.log_file, "w") as file:

file.write("========== SWITCH BROADCAST LOG ==========\n")

def add_port(self, host_id):

if host_id not in self.active_ports:

self.active_ports.append(host_id)

print(f"{host_id} connected to switch.")

else:

print(f"{host_id} is already connected.")

def remove_port(self, host_id):

if host_id in self.active_ports:

self.active_ports.remove(host_id)

print(f"{host_id} disconnected from switch.")

else:

print(f"{host_id} is not connected.")

def broadcast(self, frame):

source = frame["source"]

data = frame["data"]

destinations = []

for host in self.active_ports:

if host != source:

destinations.append(host)

print("\n========== BROADCAST ==========")

print("Source:", source)

print("Frame :", data)

if destinations:

print("Frame sent to:")

for host in destinations:

print(" ->", host)

else:

print("No other active hosts.")

# Write operation to log file

with open(self.log_file, "a") as file:

file.write("\nBroadcast Operation\n")

file.write(f"Source Host: {source}\n")

file.write(f"Frame: {data}\n")

file.write(

"Destination Hosts: " +

", ".join(destinations) + "\n"

)

# Create switch

switch = StarSwitch()

# Add multiple hosts

switch.add_port("Host1")

switch.add_port("Host2")

switch.add_port("Host3")

switch.add_port("Host4")

print("\nActive Ports:")

print(switch.active_ports)

# Broadcast frame from Host1

frame1 = {

"source": "Host1",

"data": "Hello from Host1"

}

switch.broadcast(frame1)

# Remove Host3

switch.remove_port("Host3")

# Broadcast another frame

frame2 = {

"source": "Host2",

"data": "Network announcement"

}

switch.broadcast(frame2)

print("\nFinal Active Ports:")

print(switch.active_ports)

print("\nBroadcast operations saved in switch_broadcast_log.txt")

Expected output:

Host1 connected to switch.

Host2 connected to switch.

Host3 connected to switch.

Host4 connected to switch.

Active Ports:

['Host1', 'Host2', 'Host3', 'Host4']

========== BROADCAST ==========

Source: Host1

Frame : Hello from Host1

Frame sent to:

-> Host2

-> Host3

-> Host4

Host3 disconnected from switch.

========== BROADCAST ==========

Source: Host2

Frame : Network announcement

Frame sent to:

-> Host1

-> Host4

Final Active Ports:

['Host1', 'Host2', 'Host4']